Hypertherm, Inc.

Etna Road, PO Box 5010

Hanover, NH  03755 USA

603-643-3441 Tel

603-643-5352 Fax

www.hypertherm.com

Service Announcement

# To:	All Authorized Hypertherm OEMs, System Integrators, Service Partners and Sales Associates

# From:	Hypertherm Mechanized Systems Team

# Date:	January 7, 2011

HPR400XD Pump Motor Drive Board (error 159)

| Summary: | A small percentage of HPR400XD customers have reported receiving an error code 159 (motor drive fault) which results in a loss of power supply operation. In several cases, troubleshooting has determined the fault to be in the 141027 motor drive board and upon replacement, normal operation resumes. Engineering has analyzed the failures of these boards, and found two supplied components were not performing to their rated specification. Failures of these components caused the 159 error. Engineering has implemented a design change to the 141027 board, which has been implemented in all shipments post December 10, 2010. The new revision of the board is Rev J. If the 159 error code is being seen on an HPR400XD shipped prior to December 10, 2010, Hypertherm recommends that normal troubleshooting first be followed as the 159 error code has multiple causes outside of the 141027 control board. For your assistance, the troubleshooting procedure from the instruction manual is copied in the details below. If the board is determined as the root cause, please follow normal return procedures to obtain replacement. Hypertherm’s goal is to provide the absolute highest quality in our products and services. We apologize for any inconvenience caused by this quality issue, and will continue to do our best to exceed your expectations in the future. |
| --- | --- |
| Details: | Hypertherm technical service received reports that some HPR400XD end users were experiencing 159 error codes during normal operation. The problem manifested itself with two primary symptoms: - A 159 error that occurs intermittently, and is able to be cleared by cycling power on the power supply. This intermittent failure often occurs immediately after system installation. - A 159 error that occurs after ongoing system usage. The error code is not able to be cleared by cycling power to the power supply, and requires replacement of the pump motor drive board. In both cases, it was found that replacement of the 141027 pump motor drive board allowed normal operation to resume. Failed boards were returned to Hypertherm, and engineering completed a comprehensive failure analysis. From the analysis, the symptoms were linked to two components on the board that were not performing to their rated specification from the supplier. Details are provided below: - A primary thermistor on the board was not performing to its rated 10Amp specification. This caused overheating and subsequent failure during normal system operation, which resulted in a permanent 159 error that requires replacement of the 141027 control board. Hypertherm has redesigned the board to accommodate a new 15Amp thermistor which has been validated to perform to the required specification. - A shunt resistor used to detect an over-current condition was not performing to its expected specification. This resulted in false detection of an error condition, resulting in intermittent 159 errors. In some cases cycling the power of the power supply would clear the error code. Hypertherm has worked with the supplier and redesigned the error detection circuit to eliminate the possibility of false errors. New boards and systems being shipped as of December 10th 2010 have incorporated all design changes. The new revision for the 141027 board with these changes fully implemented is Revision J. If a customer is experiencing 159 errors, Hypertherm strongly recommends that normal troubleshooting procedures be followed first, as this error code has multiple causes outside of the 141027 control board. This full procedure is in the power supply instruction manual and also included below. If the board is determined to be the root cause, please follow the normal Hypertherm return policy to obtain a replacement board. |

159 Error Code Troubleshooting Procedure: